9606EX Thermo-Hygrometer 溫濕度計

SPECIFICATION:

Temperature Range	-50 ~ +70 °C (-58 °F ~ +158 °F)

Resolution 			0.1

Accuracy 			± 0.5 °C (0 ~ +40 °C),

±1 °C (The remainder range)

Humidity Range 	10 % RH to 95 % RH

Resolution 			1 %

Accuracy 			± 3 % (30 % RH ~ 80 % RH),

± 5 % (The remainder range)

LCD Display Size 	95 x 25 mm

Dimension 			140 x 60 mm

°C/°F by Soldering Options

Low Battery Indicator

Wall Mounting & Table Standing

Battery Size AAA 1.5V x 2 (Included)

External Combination Probe with 1 M cable for temperature and humidity